A M I

RESISTANCE AND PROPULSION ANALYSIS

for

SURFACE PROPELLER APPLICATIONS

ARNESON MARINE, INC.

Version 1.1

February 1989

Copyright (C) Arneson Marine Inc, 1989

This program has been developed for the exclusive use of Arneson

Marine customers.  Arneson Marine Inc. makes no claims concerning the

validity or accuracy of the results of this program, and takes no

responsibility for any consequential loss or damage as a result of its

use.

For application assistance, please contact your Arneson Marine dealer

or an Arneson Marine Sales Office.

Main Office: 29485 Airport Road

Eugene, Oregon 97402

(503) 461-2223   fax: (503) 461-2224

telex 171 242 ARNESON DRIVE

Eastern Sales Office: 2850 N.E. 187th St.

North Miami Beach, Florida 33180

(305) 935-1336   fax: (305) 932-4673

The runtime module BRUN30.EXE is copyright (C) Microsoft Corp.

The library file ADVBAS.EXE is copyright (C) Thomas Hanlin III.

AMI is written by Paul Kamen, who may be contacted at:

1819 Addison Street, #3

Berkeley, CA 94703

(415) 540-7968   fax: (415) 540-6324

To print these instructions, type PRINTDOC at the DOS prompt.

TABLE OF CONTENTS

Quick Start  . . . . . . . . . . . . .

Overview . . . . . . . . . . . . . . .

Data Entry . . . . . . . . . . . . . .

Output Options . . . . . . . . . . . .

Interpreting the Results . . . . . . .

Resistance and Engine Files  . . . . .

User Registration Form . . . . . . . .

QUICK START -----------------------------------------------------

Three files are required to run AMI: AMI.EXE,

BRUN30.EXE, and ADVBAS.EXE.  Copy all three files

to the directory you would like to work in or to

the default drive, and type "AMI" to begin.

Additional files - TESTBOAT.AMI, TESTFILE.EHP, and

*.ENG files are input, resistance, and engine files

that may be called from within the program if

desired to confirm proper operation.  This is the

USER.DOC file, and is displayed by means of the

HELP.BAT batch file and READ.COM, a file reading

utility.  The file PRINTDOC.BAT sends the USER.DOC

file to the printer.

The program begins in the resistance screen mode.

Use right/left cursor to move to and from the

propulsion screen mode.  Numeric keys, enter key,

and up/down cursor keys control data input.  The

slash key < / > accesses the command menus.

All data entry is in English units, speeds are in

knots, displacement is in pounds.

OVERVIEW ----------------------------------------------------

AMI performs two basic calculations: Estimates the

resistance of a planing vessel using a modified

"short form" Savitsky method, and estimates the

performance of a surface-piercing propeller of

specified dimensions.   Various additional factors

are computed.  These include:

* Lift and drag from trim tabs, if fitted.

Lift force and moment are used to modify

calculated trim and resistance.

* An empirical wave drag term for lower speed

applications.  This is the "M-factor"

developed by Blount and Fox.

* Air resistance, based on estimated

frontal area and a drag coefficient of 0.6

* Appendage resistance, based on the dimensions

of the skegs of Arneson Surface Drives.

* Aeration correction, which is a reduction of

frictional resistance to account for the

"drying out" of the full length of the chine

for certain high-speed applications.

* Frictional wake fraction.  This is based on

the calculated displacement boundary layer

thickness at the transom, and is assumed to

be dissipated across the propeller disk for

an average effective value.

AMI does not directly compute the estimated speed

or optimal propeller dimensions.  It can only

analyze a specified combination of vessel, engine,

propeller, and drive parameters.  There is

necessarily a great deal of "intelligent" guesswork

required to arrive at an optimal combination of

these elements.  See the section "Interpreting the

Results" for more information.

DATA ENTRY -----------------------------------------------------

The input area of the screen is the left-hand

column. To input a value, move the highlight cursor

to the label for the desired parameter, and key in

the number (or file name).  The enter key (or

return key or "down and right arrow" key on some

keyboards) will cause the new value to be accepted

by the program.  An additional keypress of the

enter key will move the cursor down one line.  The

up and down cursor control keys also move the

cursor, but are not active while entering a value.

The output area is the right-hand column.  This will

show mostly zeros until sufficient parameters have

been defined to perform the required calculations.

Results are updated immediately whenever a new

value is input.  A click is generated when the

recalculation is complete.  Note that on PC or XT

class machines, it is advisable to wait for the

click and the display update before entering

additional keystrokes.  Display update will also be

slow on a floppy drive machine when using .EHP or

.ENG files, which must be read from disk every time

the propulsion calculations are performed.

In this version of AMI (version 1.1), all data

entry is in English units, speeds are in knots, and

all output is in English units.  A detailed

description of input parameters follows.

Vessel Speed: Speed in knots (1 knot = 1.1508 MPH)

LBP: "Length Between Perpendiculars, in feet.

For small craft, use estimated waterline

length.  This value is not used in any of the

calculations except speed/length,

displacement/length, and Froude number

outputs, which are for information only.

LCG: Distance from the bottom corner of the

transom to the Longitudinal Center of

Gravity, in feet.  If there are transverse

steps or a pin-tail or re-entrant transom, it

is necessary to estimate the location of an

"equivalent" straight single transom.

Displacement: Total vessel weight, in pounds (2204.6

lb/metric tonne)

Chine beam: Maximum chine beam, in feet. Include chine

spray strakes which have the effect of

widening the planing surface

Deadrise: Deadrise angle in degrees, at the location of

maximum chine beam.  The angle should be from

a point on the keel at centerline to the edge

of the chine strake.  (Therefore a boat with

a rounded section at the keel and large spray

strakes would have an effective deadrise

angle several degrees less than the actual

angle of most of the bottom surface.)

Air drag area: Cross sectional area, in square feet, of the

boat above water in operating attitude. Used

to calculate air resistance.

No. Shafts: Number of propeller shafts.

ASD skeg model: Model number of Arneson Surface Drive.  Used

only to determine skeg dimensions and

appendage drag.  The input must be

0,6,8,10,12,14,16,or18.  If 0, then appendage

drag will be 0.

No. Hulls: Number of hulls. Input 2 for a catamaran.

No. flaps: Number of flaps or trim tabs.

Flap span: Span or transverse  dimension of each flap or

trim tab, in feet.

Flap Length: Longitudinal or fore-and-aft dimension of

each flap or trim tab, in feet.

Flap Location: Distance from bottom corner of transom to

forward edge of flap or trim tab.  For a trim

tab hinged at the transom, enter 0.  For a

wedge with trailing edge at the transom,

input flap location equal to flap length.

Flap angle: Angle of flap or trim tab deflection, in

degrees, from the angle that produces 0

lift force.

Drive Trim: Trim angle of the Arneson Surface Drive, in

degrees down from the nominal 6 degree

installation.  (A trim angle of -1, for

example, means one degree of up-trim from the

nominal neutral position, or 5 degrees

relative to the boat bottom.)  This is used

to compute propeller submergence, and has no

direct effect on vessel trim.

Prop. Diameter: Propeller diameter, in inches.

Prop. Pitch: Propeller pitch, in inches.

No. of Blades: Number of propeller blades, 3 to 6.

Red. Gear Ratio: Reduction gear ratio.  Do not input the ":1"

which the program appends to the display.

Engine RPM: Engine RPM in the operating condition. If

Engine RPM is input as a lower value than

Rated RPM, then the program will compute a

reduced power output based on a generic

engine curve (unless an engine file is

called).

Rated HP: Rated engine power, in mechanical horsepower

of 550 ft-lb/sec, at the engine's rated RPM.

Rated RPM: RPM at which the Rated HP applies.

% Ambient Losses: Reduction of power from engine rating due to

environmental conditions.  Defaults to 5%.

% Trans. Losses: Transmission and gear losses. Defaults to 2%.

ASD model: Model number of Arneson Surface Drive.  Used

to determine relationship between drive trim

angle and propeller submergence.  The input

must be 0,6,8,10,12,14,16,or18.

Engine input: Normally left blank, unless an engine file is

called for input.  See the section of these

instruction on file input.

EHP input: Normally left blank, unless and EHP or

resistance file is called for input.  If a

numeric value is entered instead of a file

name, then that value is used for bare hull

resistance.  Appendage and air drag from the

resistance mode are added to obtain total

resistance.

Input Files: Input data may be saved to and recalled from a disk

file.  Entering a slash (/) in either of the screen

modes will access the command menu, and then

entering "F" (upper or lower case) will bring up

the "Recall or Save" prompt.  When asked for a file

name, enter a DOS-legal file name (8 or fewer

characters beginning with an alpha character, no

spaces).  The extension .AMI will be added

automatically to both input and output entries.

OUTPUT OPTIONS --------------------------------------------------

Output from AMI can be presented in 5 different

ways, as summarized below.  Note that when a file

name is requested, the input should be a legal DOS

file name without the extension.  The extension

.TBL will be added automatically for all output

files.

1) On-screen, interactive.  This is the default

mode.  Results appear on the left-hand side

of the screen, updated as parameters are

input.

2) On-screen display of resistance and EHP

table.  From the resistance screen mode, type

"/" to enter the command menu, then "D" (for

"display table").

3) Print resistance optimization table, single

parameter.  From the resistance screen mode,

type "/" for the command menu, then "S" for

single-parameter table.  The prompt line

(bottom line of the screen) will ask for

"parameter number,"  meaning the number to

the left of the on-screen input parameter

label.  The prompt line will then ask for low

value, high value, and increment of the

selected parameter.  The optimization table

output can be directed to the printer or to a

disk file.  Enter a file name at the file

name prompt.

4) Print resistance optimization table, double

parameter.  From the resistance screen mode,

type "/" for the command menu, then "D" for

double-parameter table.  The prompt line

will ask for "first parameter number,"

followed by prompts for low value, high

value, and increment.  Repeat at the prompts

for the second parameter.  The resulting

optimization table will show resistance,

vessel trim angle, and EHP for every

combination of the two selected parameters.

Note that for printed output, only seven

values of the second parameter will be shown

because of the width limitation.  There is no

limit to the number of values for the first

parameter, or to either parameter for file

output.

5) Print propulsion analysis.  From the

propulsion screen mode, type "/" for the

command menu, then "P" for the "printer or

file" prompt.  The output is a summary of

resistance and propulsion screen input and

output values for the currently displayed

condition.

INTERPRETING THE RESULTS ----------------------------------------

A summary of the output values follows:

Speed/length ratio: Speed in knots divided by square root of LBP

in feet.  This is the traditional quasi-non-

dimensional speed index in English units.

Vol. Froude No.: Speed divided by sixth root of volumetric

displacement and the square root of

gravitational acceleration.  This is a non-

dimensional speed coefficient useful for

evaluating planing hulls.

Beam Froude No.: Speed divided by square root of beam and

square root of gravitational acceleration.

Another non-dimensional speed coefficient.

Length Froude No.: Speed divided by square root of LBP and

square root of gravitational acceleration.

Yet another non-dimensional speed

coefficient.

Disp/length ratio: Displacement in long tons (2240 lb) divided

by the cube of LBP/100.  This is a quasi-non-

dimensional index of displacement compared to

length, in English units.

Wetted length, mean: Average wetted length of bottom in planing

condition, in feet.  As the mean wetted

length approaches or exceeds LBP, the

accuracy of planing resistance calculations

will be degraded.

Wetted length, keel: Wetted length of keel in planing condition,

in feet.  Experience indicates that a

calculated keel wetted length somewhat

greater than the LBP is acceptable.

Wetted length, chine: Wetted length of chine in planing condition,

in feet.  Will be zero if chine is

theoretically dry.

Immersed chine depth: Depth of aft end of chine below nominal water

surface in planing condition, in feet.  A

negative value indicates that the chine is

above the nominal water surface.

Wetted beam, max.: Theoretical wetted beam at transom, in feet.

Equal to maximum chine beam if chine wetted

length and immersed chine depth are positive.

Trim angle (deg.): Vessel trim angle in planing condition, in

degrees.

Flap lift: Total lift produced by flaps or trim tabs, in

pounds.

Flap drag: Total drag produced by flaps or trim tabs, in

pounds.

Bare hull drag (lb): Drag in pounds computed by Savitsky

equations, without additional drag sources.

(Bare hull drag does reflect the effect of

flap lift on drag.)

M factor drag: Additional drag in pounds attributable to

wave-making resistance, applicable at lower

speeds and higher displacements.  Probably

conservative for fine-entry hull forms.

Air drag: Air resistance in pounds based on input air

drag area and speed.  Drag coefficient is

0.6, air density is 0.00238 slugs/ft^3.

Appendage drag: Drag in pounds caused by the fin or skeg of

Arneson Surface Drives.

Aeration Correction: Reduction of drag in pounds caused by partial

drying out of the bottom.  Only applicable

when chine wetted length is zero, and bottom

strakes are in effective locations.

Total drag (lb): Total of all above sources of drag, in

pounds.

EHP: Effective horsepower, in units of

550 ft-lb/sec. (1 KW = 1.34124 HP)

Nominal Prop. Slip: Difference between propeller pitch times

propeller RPM, which can be represented as a

speed, and actual vessel speed.  Expressed in

per cent.

Frict. Wake Fraction: Velocity of water accelerated by vessel,

expressed as percentage of vessel speed.

Based on frictional boundary layer only;

there may be additional wake fraction due to

the flow field around the hull.

Advance Velocity: Velocity of propeller through water, in

knots, after correcting for wake fraction.

Propeller Slip: Actual propeller slip based on advance

velocity instead of vessel speed.

Propeller P/D: Propeller pitch/diameter ratio.

Rated Torque: Engine torque, in lb-ft, based on input rated

power and rated RPM values.  (Note that for

most engines, actual maximum torque at

reduced RPM can be greater than torque at

maximum ratings.)

Shaft RPM: Engine RPM divided by reduction ratio.

Available HP: Power available at the propeller after

applying loss factors.

Cavitation No.: Non-dimensional index of propeller cavitation

at the 0.7 radius.

% Submerged area: Submerged area of propeller disk, in per

cent.  A nominal Arneson Surface Drive

installation has 50% submerged propeller area

in normal operating condition.

J: Propeller advance coefficient.

Kt: Non-dimensional propeller thrust coefficient,

interpolated from propeller curves.

Kq: Non-dimensional propeller torque coefficient,

interpolated from propeller curves.

Torque Req./Shaft: Torque absorbed by propeller, in lb-ft.

Power Required/Shaft: Power absorbed by propeller, in HP.

Thrust/Shaft: Thrust produced by propeller, in lb.

Total Thrust: Thrust/shaft times number of shafts.

Propeller Efficiency: Propeller efficiency, interpolated from

propeller curves.

EHP/Rated HP: Effective Horsepower compared to rated engine

power.

Excess Thrust: Total thrust produced in excess of total

vessel resistance, expressed in per cent.  If

negative, there is insufficient thrust

for the input vessel speed.

Excess Power: Available power, in excess of power absorbed

by propeller, expressed in per cent.  If

negative, there is insufficient power to turn

propellers at input conditions.

Thrust/Power Margin: Excess thrust times excess power.  Represents

net thrust/power margin, or service allowance.

HP/L.Ton: Rated power in HP divided by displacement in

long tons (1 long ton = 2240 lb).

f(HP,Disp,LBP): Estimated maximum speed of the vessel based

only on power, displacement, and LBP.

Sometimes useful for preliminary estimates.

CAVEATS: Users should be aware of the following potential

problem areas when using AMI:

When calculated wetted length approaches or exceeds

vessel length, the validity of the planing resistance

analysis deteriorates.  The Savitsky equations may be

assuming that there is more length of hull available

to provide the necessary lift and buoyancy than

actually exists.  Experience has shown that it is

usually acceptable for the calculated keel wetted

length to exceed actual keel length, but the mean

wetted length should not exceed LBP.

There is no upper limit for trim tab or flap angle of

attack.  Use angles above 10-12 degrees with caution.

Propeller curves can sometimes indicate highest

efficiency at excessively high slip levels,

especially for applications with marginal reduction

ratio.  Blindly pursuing the highest efficiency

could lead to unacceptable performance at speeds

off the design condition.  Propeller slip should be

carefully evaluated based on experience.

The program appears to exaggerate the effect of drive

trim and varying propeller submergence.  Avoid down-

trims of more than about 2 degrees, and avoid up-

trims of more than about 3 degrees.  Larger values of

drive trim may be used, however, to evaluate transom

shapes or other installation anomalies which produce

significant variations in propeller submergence.

Vertical force due to shaft inclination and the

vertical component of propeller side force are not

considered when computing vessel trim.  For a high

power, high deadrise application with outboard

turning propellers, this vertical force can have a

significant effect.  Actual vessel trim angle,

therefore, could be less than the computed trim

angle.

RESISTANCE AND ENGINE FILES -------------------------------------

This version of AMI allows vessel EHP and

resistance curves to be read directly from a file

rather than computed by the program.  Engine power

versus RPM curves and torque versus RPM curves can

also be read in place of the generic engine power

curve algorithm contained in the program.  Files

are called by entering the file names in the last

two input rows of the propulsion screen.  Use a

legal DOS file name (up to eight characters, first

character must be alphabetic, no blanks) but do not

include a file extension.  The extension ".EHP"

will be automatically added to EHP or resistance

files, and the extension ".ENG" will be added to

engine torque or power curve files.

.ENG and .EHP files can be created using any modern

word processing program.  Make sure the file is

saved as an ASCII or "non-document" file.  When

creating the file, you must include the

appropriate extension.  The contents of a file can

be checked with the READ.COM utility.  From the

DOS prompt type "READ" followed by the full file

name, including extension.  The formats of the .ENG

and .EHP files are described below:

.ENG format - Line 1: The first line is a title, read but not

output. do  not include any commas on this

line.

Line 2: The word "POWER" (with no punctuation)

indicates that the file contains power

data, in the units of mechanical horsepower

(550 ft-lb/sec).  The word "TORQUE"

indicates that the file file contains

torque data in units of lb-ft.

Line 3: Should be "0,0" to indicate 0 power or 0

torque at 0 RPM.

Subsequent Lines: Two numbers separated by a comma.  The

first is RPM, the second is power or

torque.  These lines define the power or

torque curve.

End of Data LIne: "-1" or any negative number, to mark end of

data.

Additional lines: Additional lines may be added after the -1

line as comments.  These lines are not read

by the program.

.EHP format - Line 1: The first line is a title, read but not

output. do  not include any commas on this

line.

Line 2: Two words separated by a comma.  "EHP" as

the first word indicates that the file is

an EHP table.  "RESISTANCE" as the first

word indicates a resistance table.  If the

second word is "BARE, then EHP or

resistance are bare hull values, and

appendage resistance and air resistance

computed from the resistance portion of the

program are added to the file data.  If

the second word is "TOTAL" then no

additions are made.

Line 3: Should be "0,0" to indicate 0 EHP or 0

resistance at 0 vessel speed.

Subsequent Lines: Two numbers separated by a comma.  The

first is vessel speed, the second is EHP

(in horsepower of 550 ft-lb/sec) or

resistance (in pounds).  These lines define

the EHP or resistance curve.

End of Data LIne: "-1" or any negative number, to mark end of

data.

Additional lines: Additional lines may be added after the -1

line as comments.  These lines are not read

by the program.

Note that when inputting resistance or reading

resistance or EHP from a file, it is still

desirable to input approximate vessel data into the

resistance mode screen.  This is the only way to

generate the wetted length required for computing

wake fraction.   Also note that the resistance value

displayed in the resistance screen mode is the

resistance calculted from vessel data input, and not

from the resistance or EHP input file.  Switching to

propulsion screen mode will display resistance as

read from the file.

USER REGISTRATION FORM--------------------------------------------

Arneson Marine is very much interested in your

comments and suggestions for future versions of AMI.

Please send the information requested on the form to:

Arneson Marine Inc.

29485 Airport Road

Eugene, Oregon 97402

- - - - - - - - - - - - - - - - - - - - - - - - - - -

A M I

User Registration Form

Company name and

mailing address: ______________________________________

______________________________________

______________________________________

Your  name: ______________________________________

Type of computer

and printer: ______________________________________

Description of

vessels analysed: _____________________________________

_______________________________________________________

_______________________________________________________

Comments: _____________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

Suggestions: __________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________